SYNTH-GPR

Synthetic Ground Penetrating Radar Dataset & ML Pipeline

Project Overview & Architecture Documentation

Author: Álvaro Jería

June 2024

# Table of Contents

1. Project Overview

2. Problem & Solution

3. Technical Architecture

4. Data Pipeline

5. Core Components

6. Key Features

7. Getting Started

8. Project Statistics

# 1. Project Overview

## What is Synth-GPR?

Synth-GPR is a comprehensive framework for generating synthetic Ground Penetrating Radar (GPR) datasets and training machine learning classifiers to assess railway ballast fouling conditions.

## Core Innovation

The key innovation is predicting fouling class from GPR waveform features alone—without relying on metadata (material composition, density, moisture). This is critical because metadata is unavailable in real-world GPR field deployments.

## Baseline Performance

| Metric | Value |
| --- | --- |
| Dataset Size | 30,000+ samples |
| Feature Dimension | 572 (waveform only) |
| Model Type | Random Forest |
| Balanced Accuracy | 70.83% |
| Training Time | ~30 minutes |

# 2. Problem & Solution

## The Problem

Traditional ballast fouling assessment relies on:

• Lab measurements—expensive, limited samples

• Metadata-based models—require lab inputs (permittivity, density, moisture)

• Hand-engineered features—no generalization to field data

## The Solution

Synth-GPR solves this by:

• Generating synthetic data at scale (10k+ samples with full geometric control)

• Extracting waveform-only features (572 features from A-scan alone)

• Training production models with baseline 70.83% balanced accuracy

• Enabling field deployment (only Ez waveform needed, no metadata)

# 3. Technical Architecture

## High-Level Overview

Synth-GPR is organized into three main layers:

| Layer | Purpose | Key Modules |
| --- | --- | --- |
| Data Generation | Create synthetic GPR geometries and run FDTD simulations | rock_packing, gpr_commands, scene_generator |
| Feature Extraction | Extract 572-dimensional features from raw A-scan waveforms | signal_processing, feature_extraction |
| ML Pipeline | Train and evaluate classifiers for fouling prediction | train_rf, train_xgboost, evaluation |

## Modular Design

The codebase is organized into reusable components:

• Domain Models (value_objects, coordinates, scene_parameters)

• Data Access Layer (repositories, filesystem_repository)

• Core Algorithms (signal_processing, rock_packing, fouling calculation)

• I/O Utilities (file_reader, file_writer, dataset_io)

• Visualization (render, panels, publication_figures)

# 4. Data Pipeline

## End-to-End Workflow

| Stage | Input | Process | Output |
| --- | --- | --- | --- |
| 1. Generation | Parameters (fouling level, antenna, material) | Rock packing algorithm + geometry creation | .in files |
| 2. Simulation | .in geometry files | gprMax FDTD electromagnetic simulation (400 MHz Ricker) | .out HDF5 files with Ez waveform |
| 3. Extraction | .out files | Feature extraction from A-scan signals | 572-dimensional feature vectors |
| 4. ML Training | Feature vectors + labels | Random Forest / XGBoost training | Fouling classifier model |

## Feature Extraction Details

| Feature Group | Count | Description |
| --- | --- | --- |
| Time-domain statistics | ~25 | Mean, RMS, std, skewness, kurtosis, peak values, area |
| Hilbert envelope | ~25 | Energy envelope metrics capturing pulse shape |
| FFT features | 8 | Frequency domain analysis (peak, bandwidth, entropy, flatness) |
| STFT features | 7 | Time-frequency energy in depth bands |
| Slice statistics | 28 | 14 temporal segments × 2 metrics |
| Grid features | 480 | 16×10 spatial-temporal grid × 3 channels |
| TOTAL WAVEFORM | 572 | All extracted from raw A-scan signal |

# 5. Core Components

## Rock Packing Algorithms

12 configurable algorithms with different speed/quality tradeoffs:

| Algorithm | Speed | Quality | Use Case |
| --- | --- | --- | --- |
| RSA (Random Sequential) | < 1s | Low | Fast prototyping |
| Grid | < 1s | Low | Regular patterns |
| Circlify | 1-5s | Medium | Testing |
| Simulated Annealing | 5-10s | Medium | Balanced |
| ShangChu (Recommended) | 15-20s | High | Production datasets |
| HybridShang | 15-20s | High+ | Enhanced quality |

## Fouling Classification

Five fouling classes defined by Fouling Index (FI) and Percentage Void Contamination (PVC):

| Class | Label | FI Range | Description |
| --- | --- | --- | --- |
| C | Clean | 0.0–0.2 | No fouling material in voids |
| MC | Moderately Clean | 0.2–0.4 | Minimal fouling |
| MF | Mixed Fouling | 0.4–0.6 | Partial void contamination |
| F | Fouled | 0.6–0.8 | Significant fouling |
| HF | Highly Fouled | 0.8–1.0 | Voids mostly filled with fines |

## Electromagnetic Simulation

Configuration:

• Antenna: 400 MHz Ricker wavelet (production standard)

• Time window: 20 ns

• PML boundary: 10 cells

• Output: Ez time-domain waveforms (A-scans)

# 6. Key Features

## Synthetic Data Generation

Generates realistic GPR geometries with configurable:

• Fouling levels (5 classes: Clean → Highly Fouled)

• Rock packing algorithms (12 variants)

• Antenna configurations (bistatic, frequency-selectable)

• Material properties (permittivity, conductivity, moisture)

## Visualization

Comprehensive visualization tools:

• High-resolution blueprint diagrams of simulated geometries

• A-scan visualization with physical GPR interpretation

• 6-panel analysis plots (raw signal, Hilbert envelope, FFT, STFT, grid, physics)

• 3D scene animations and publication-quality figures

## Machine Learning Pipeline

Production-ready ML capabilities:

• Random Forest and XGBoost classifiers

• Class balancing and hyperparameter tuning

• Cross-validation and rigorous model evaluation

• Feature importance analysis and confusion matrices

# 7. Getting Started

## Installation

Prerequisites: Python 3.8+, gprMax, pip

1. Clone repository and navigate to directory

2. Create virtual environment: python -m venv venv

3. Install dependencies: pip install -r requirements.txt

4. Verify setup: pytest tests/ -v

## Quick Start Pipeline

Step 1: Generate Synthetic Geometries

python scripts/pipeline/generate_in_files.py output/ --mode batch --labels CL MC MF F HF -n 50 --angular

Step 2: Run Electromagnetic Simulations

python scripts/pipeline/run_simulations.py output/ -j 4

Step 3: Extract Features

python scripts/pipeline/extract_features.py output/ --output features.csv

Step 4: Train ML Classifier

python scripts/pipeline/train_rf_waveform_only.py

# 8. Project Statistics

## Codebase Overview

| Component | Count/Details |
| --- | --- |
| Python Modules | 60+ files in src/ |
| Scripts | 30+ executable scripts |
| Test Files | 25+ test modules |
| Documentation Files | 80+ markdown files |
| Packing Algorithms | 12 variants |
| Feature Groups | 7 categories (572 total features) |
| Fouling Classes | 5 (Clean to Highly Fouled) |

## Directory Structure

src/ — Core library modules

• domain/ — Domain models and value objects

• repositories/ — Data access layer

• visualization/ — Plotting and rendering

• [60+ modules] — Core algorithms and utilities

scripts/ — Executable scripts

• pipeline/ — Main generation and ML workflow

• analysis/ — Validation and research analyses

• experiments/ — Research experiments

• visualization/ — Plotting and rendering scripts

docs/ — Comprehensive documentation

• setup/ — Installation and configuration

• architecture/ — Technical architecture

• algorithms/ — Algorithm documentation

• research/ — Research papers and analyses

tests/ — Test suite (pytest)

## Dependencies

| Category | Key Libraries |
| --- | --- |
| Scientific Computing | numpy, scipy, pandas |
| Machine Learning | scikit-learn, xgboost |
| Visualization | matplotlib, plotly, pillow |
| Data I/O | h5py (HDF5), pyarrow (Parquet) |
| Testing | pytest, hypothesis |
| Configuration | pyyaml, python-dotenv |

## Development Standards

The project follows professional development practices:

• Code style: Black formatter, type hints, docstrings

• Testing: Comprehensive pytest suite with unit and integration tests

• Documentation: Markdown files in docs/, inline docstrings

• Version control: Git with clear commit history

• CI/CD ready: Configuration for automated testing and validation

# Summary

Synth-GPR is a mature, well-engineered framework for synthetic GPR dataset generation and machine learning-based fouling classification. Key strengths include:

✓ Waveform-only feature approach enables field deployment

✓ Modular architecture with 12 packing algorithms and 7 feature groups

✓ Production-ready ML pipeline with baseline 70.83% accuracy

✓ Comprehensive documentation (80+ markdown files)

✓ Rigorous testing and professional development standards

✓ Scalable infrastructure for generating 10k+ synthetic samples

For questions, issues, or collaboration: alvaro.jeria.m@gmail.com